# Packet Tracer Troubleshooting Lab: "Nothing Pings"

Fault Log

Name: Adams Selah Mensah

Course: BSc Computer Science, Central University

Submitted file: Lab_Fixed_AdamsSelahMensah.pkt

# 1. Summary

The inherited office network (Sales VLAN 10, Admin VLAN 20, Server LAN) had seven faults across physical/admin state, Layer 3 addressing, and Layer 2 VLAN and trunk configuration. Each was found using show commands, ping and the running configuration, then corrected. After the fixes, all success criteria were met.

# 2. Faults found and fixed

| # | Device | What was wrong | How it was found | Fix |
| --- | --- | --- | --- | --- |
| 1 | R1 Gi0/1 | Interface was administratively shut down. | show ip interface brief showed "administratively down / down". | no shutdown on Gi0/1. |
| 2 | R1 Gi0/1 | IP address was 192.168.100.2; the design gateway for the server LAN is 192.168.100.1. | Compared show ip interface brief output with the design table. | ip address 192.168.100.1 255.255.255.0 |
| 3 | SW1 Fa0/2 | Port was in VLAN 10; PC2 (Admin) belongs in VLAN 20. | show vlan brief listed Fa0/1 and Fa0/2 both in VLAN 10, with VLAN 20 empty. | switchport access vlan 20 |
| 4 | SW1 Gi0/1 | Trunk allowed only VLAN 10, so VLAN 20 traffic was blocked. | show interfaces trunk listed only VLAN 10 as allowed. | switchport trunk allowed vlan 10,20 |
| 5 | PC1 | Default gateway was 192.168.10.254 (does not exist); should be 192.168.10.1. | Compared PC settings with the design table; ping to PC2 failed while ping to 192.168.10.1 worked. | Set gateway to 192.168.10.1. |
| 6 | PC2 | IP address was 192.168.2.10 (wrong subnet); should be 192.168.20.10. | Compared PC settings with the design table; ping to its gateway 192.168.20.1 failed. | Set IP to 192.168.20.10. |
| 7 | R1 Gi0/0.20 | Subinterface used encapsulation dot1Q 200; Admin VLAN is 20. | show running-config: the dot1Q tag did not match the subinterface/VLAN number, although the interface showed up/up. | encapsulation dot1Q 20 |

# 3. Verification

After the fixes, the following were confirmed:

- PC1 pings its gateway (192.168.10.1).

- PC2 pings its gateway (192.168.20.1).

- PC1 pings PC2.

- PC1 and PC2 ping Server0 (192.168.100.10).

- All R1 interfaces (Gi0/0, Gi0/0.10, Gi0/0.20, Gi0/1) show up/up in show ip interface brief.

- show interfaces trunk shows VLANs 10,20 allowed, active and forwarding; show vlan brief shows Fa0/1 in VLAN 10 and Fa0/2 in VLAN 20.

# 4. Key lesson

Fault 7 showed that an interface can be up/up while the configuration is still wrong: the dot1Q tag on Gi0/0.20 did not match the VLAN, so Admin traffic was ignored.

Hence, interface status alone does not prove a link works, so connectivity must be tested with ping and the configuration compared against the design.